Review

- Fingerprints aka Friction Ridge Impressions

  - FRI includes toe prints.

- Core-Delta relationship would help easily define the class of a fingerprint.

  - Left loop, right loop, whorl, plain arch, tented arch, twin loop

- Methods of fingerprinting

  - Off-line: Inked, powder dust

  - On-line: ultrasonic, thermal, optical(direct imaging), capacitive

- Compared to inking and FTIR fingerprints, quality of latent fingerprint is much lower.

- Minutiae-Based Representation (level 2)

- Fingerprint Representation

  - Local ridge characteristics(minutiae): ridge ending and ridge bifurcation

  - Singular points: Discontinuities in ridge orientation

- Real World - FBI Example

  - Lights Out Mode: machine makes the final determination. No human examiner is involved.

  - Lights On Mode: A machine is used to receive a candidate list. Depending upon the procedure, a human examiner is used to make the final determination.

- Number of cores should be equal to the number of deltas.

- Minutiae point characterized by 3 values: x, y, and Θ.

- Minutiae Extraction: Input Image > Orientation Estimation > Ridge Filter > Ridge Thinning Minutiae Detection > Post Processing > Minutiae Extraction

- View image as matrix. Binary matrix. Full of 0s and 1s. A 0 is a dot, a 1 is a bright pixel. If there is a 3x3 mask, and slide it over a matrix. If you check the middle of the matrix as you move the mask, if the middle is a zero,

  - Ridge ending: 2

  - Ridge Bifurcation: 3

- 3 Factors affecting fingerprint recognition

  - Pressure

  - Non-uniform deformation

  - Rotation and translation

- Intra-class Variation

  - Lead to high False Non-Match

- RANSAC (Random Sample Consensus)

  - Match minutiae points of two fingerprint scans