- WHY NIR - Near Infrared - Optics?

  - Dark-color Iris: The textural details of dark-colored irides (majority of the world population) are more evident in the NIR channel than in the red, green, or blue channels.

  - Non-intrusive: NIR light cannot be perceived by the human eye. This ensures that the image acquisition process is non-intrusive, even when the eye is required to be in close proximity to the sensor and the NIR light source.

- Structure of the iris

  - The posterior layer at the back, which is two cells thick, contains heavily pigmented epithelial cells, making it impenetrable to light.

  - The muscle layer above it consists of the sphincter (circle-like) and dilator (spoke-like) muscles that contract and dilate the pupil.

  - The stromal layer, located above the muscles, is made up of collagenous connective tissue (arranged in an arch-like configuration) and blood vessels (arranged along the radial direction).

  - The anterior border layer is the foremost layer and has an increased density of chromatophores (i.e. pigment containing cells) compared to the stromal layer.

- Applications: past and present

  - Aadhaar Program - India

  - National ID Program - Indonesia

  - National Population register (“RENAPO”) - Mexico

  - CAC Program - US Department of Defense

  - Privium Program - Schiphol Airport, Netherlands

  - Border Control System - UAE

  - NEXUS and CANPASS Program - Canada

- Iris Sensors

- Factors Negatively Affecting an IRIS image:

  - Occlusion

  - Motion Blue

  - Defocus

  - Non-uniform illumination

  - Poor resolution

- Quality Assessment Techniques

  - Examination of sharpness of the portion between pupil and iris from the image

  - Computation of energy of the image from high spatial frequencies using Fourier analysis

  - Quantifying the energy from 2D wavelets at local concentric iris bands

- Iris Segmentation

  - Definition: determining the pixels in an iris image that correspond to the iris

  - Challenges:

    - Iris texture is stochastic and irregular, precluding appearance-based algorithms

    - Limbus and pupillary boundaries may not be sharp

    - Eyelids and eyelashes may create irregular boundaries or be detected as spurious edges

- Integro-Differential Operator (IDO)

  - Assumptions:

    - Limbus and pupillary boundaries are circular

    - The magnitude of the limbus and pupillary boundary edge pixels is stronger than any other circular edge in the image

  - Summary

    - Radial Gaussian filter: convolved across image to smooth out crypts, furrows, etc.

    - Circular Integral: gradients are summed across a candidate circle’s boundary

    - Circle with max gradient sum is assumed the pupillary boundary

    - Circle with next highest gradient across vertical edges is assumed limbus boundary

- Geodesic Active Contours (GAC)

  - Assumptions

  - Limbus boundary may not be circular (e.g. due to occlusions)

  - Geodesic active contours can be used to evolve a non-circular limbus boundary approximation

  - § Summary

    - Initial contour is placed just outside pupillary boundary

    - Image gradients used to create a “stopping function” image as a stopping condition

    - Contour is evolved outward using an update function until it reaches stopping condition

    - Contour may split at local minima and later remerge, preventing strong edges in iris texture from stopping the GAC evolution

- Iris Image Enhancement

  - Normalized iris images may undergo enhancement techniques for better representation

- Iris Normalization

  - Challenge with Iris

    - Size of iris and pupil varies greatly (e.g. dilation/contraction of pupil and distance to sensor)

  - Solution

    - “Normalize” the iris by unwrapping it into a rectangular region

  - Daugman’s rubber sheet model used to account for variations in iris size caused by

    - dilation and constriction of pupil

    - distance from the camera during image acquisition